三维重建及快速成型技术在室间隔缺损手术中的应用

摘 要

背景

3D打印技术出现于20世纪80年代中期，它是一种以数字模型文件为基础，运用粉末状金属或塑料等可粘合材料，通过逐层打印的方法来构造物体的技术。3D打印技术与医学相结合，由临床需求为导向从而衍生出来的一系列医学模型是近年来医学领域的新突破。

目的

基于256CT数据行室间隔缺损患儿病变部位三维重建并制作患儿的室间隔缺损的实体模型，探讨其在小儿室间隔缺损修补手术中的应用价值，为临床室间隔缺损修补术提供崭新的诊疗视角。

方法

选取2014年9月至2015年7月郑州市心血管病医院收治的经彩色超声确认的室间隔缺损患儿30例，男17例，女13例，年龄2岁5个月~3岁，平均年龄2岁8个月，体重9.6±2.6kg。30例室间隔缺损患儿均行增强256CT心脏影像学检查，设备由郑州市心血管病医院提供，设备型号为荷兰Philips Brilliance iCT，层厚0.45mm，获得256CT连续断层平扫医学数字成像和通信 (Digital Imaging and Communications in Medicine, DICOM)图像数据，然后将得到的DICOM图像数据导入到比利时Materialise公司医用Mimics19.0软件，优化图像，以减少256CT因素及呼吸运动、心脏跳动产生的阴影、斑点。根据CT成像原理确定心脏造影剂、心肌、脂肪的阈值范围，并标示。利用医用Mimics软件中的再利用区域生长及动态区域生长算法对目标区域进行分割。而后利用Mimics软件编辑蒙面，利用编辑命令功能进行精细分割，分离内部造影剂，保留正常组织结构及病变部位图像。得到30例患儿的三维重建数据，依据三维重建数据利用美国3DP Unlimited X1000工业打印机打印出3D模型。

结果

30例行三维重建及快速成型技术指导室间隔缺损修补手术患儿均取得理想的CT原始数据，而后根据医用Mimics软件处理所得的数据进行3D打印。术中所见与术前三维重建模型的病灶结构变化及室间隔缺损大小一致。术前三维重建模型测量室间隔缺损大小平均直径为7.10±0.2mm，术中实测室间隔缺损平均直径为7.20±0.2mm，两者比较差异无统计学意义（t=0.82,P=0.43）。30例患儿均按手术前手术规划完成手术，手术过程顺利，室间隔缺损补片均缝合良好，补片形态、大小满意。30例患儿均未出现房室传导阻滞及三尖瓣功能异常等并发症。左右流出道形态正常，未出现流出道梗阻改变及室间隔修补术后残余漏。

结论

1.通过三维重建及快速成型技术可以获得病理器官的三维模型，包含心脏腔室内部结构、心脏房室壁及心脏血管解剖结构关系。

2.根据三维模型，可以完成病变部位任意角度观察及测量、便捷术前沟通、规划手术路径、设计手术方案、指导室间隔缺损手术、对比手术效果等。

3.三维重建及快速成型技术还可以对法洛氏四联症、右心室双出口、主动脉夹层等复杂心血管疾病提供诊断全新视角，辅助手术，降低手术时间、提高手术成功率。

关键词 三维重建；快速成型技术；256CT；室间隔缺损

Clinical application of three dimensional reconstruction and rapid prototyping in the repair of children with ventricular septal defect surgery

Abstract

Background

3D printing technique appeared in the mid-20th century, 80 years, it is a kind of digital file-based model, the use of powdered metal or plastic bondable material, layer by layer by the method of printing technology to construct the object. Combined with 3D printing technology and medicine, the demand for the purpose that a series of clinical medicine derived from the model is a new breakthrough in the medical filed in recent years.

Objective

Based on the data line 256CT ventricular septal defect in children lesion reconstruction and create a solid model of children with ventricular septal defect, ventricular septal defect in children to explore the value of repair surgery, providing a new treatment for clinical ventricular septal defect repair perspective.

Methods

30 children with ventricular septal defect selected in September 2014 to July 2015, Zhengzhou City Hospital of cardiovascular disease by color Doppler ultrasound confirmed, 17 males and 13 females, aged 2 years, 5 months to 3 years, mean 2 years and 8 months of age, weighing 9.6 ± 2.6kg. 30 cases of children with ventricular septal defect underwent cardiac imaging enhancement 256CT, equipment provided by the Cardiovascular Hospital of Zhengzhou City, device models for the Dutch Philips Brilliance iCT, thickness 0.45mm, continuous tomographic scan obtained 256CT Medical Digital Imaging and Communications (Digital Imaging and Communications in Medicine, DICOM) image data, and then DICOM image data obtained into the Belgian company Materialise Mimics19.0 medical software, image optimization to reduce 256CT factors and respiratory movement, heart beat shadows and spots. Determine cardiac imaging agents, cardiac muscle, fat threshold range based on CT imaging principle, and labeling. Medical use Mimics software reuse region growing and dynamic region growing algorithm to segment the target area. Then use Mimics software to edit masked , use the Edit command to be finely divided, separate internal contrast agent, to retain the normal structure and lesion images. Three-dimensional reconstruction data obtained 30 cases of children, according to the three-dimensional reconstruction data using US industrial 3DP Unlimited X1000 printer to print out 3D models

Results

30 routine three-dimensional reconstruction and rapid prototyping of technical guidance ventricular septal defect in children undergoing repair are to achieve the desired CT raw data and then print a 3D Mimics software based on medical treatment of the resulting data. Intraoperative findings consistent with structural changes in the preoperative tumor model three-dimensional reconstruction and ventricular septal defect size. Preoperative three-dimensional reconstruction model measuring ventricular septal defect size average diameter of 7.10 ± 0.2mm, ventricular septal defect surgery found a mean diameter of 7.20 ± 0.2mm, the difference was not statistically significant (t = 0.82, P = 0.43). 30 cases of children according to pre-operative surgical planning to complete the surgery, the surgery went smoothly, ventricular septal defect patch are stitched good patch shape, size satisfaction. 30 cases of children were not there atrioventricular block and tricuspid valve dysfunction and other complications. Left and right outflow tract morphology normal outflow tract obstruction and interventricular septum does not appear to change the residual shunt after repair.

Conclusions

Three dimensional reconstruction and rapid prototype technology of the lesions in the patients with ventricular septal defect based on 256CT data can accurately reflect the type of ventricular septal defect、defect size, providing the foundation of evaluations before cardiac surgery and surgical rehearse, intraoperative guidance, postoperative evaluation.

Key words Ventricular septal defect; Three dimensional reconstruction；Rapid prototype technology；256CT